Notes for Tables 36, 37 and 38:

1. See Figure 1 for sampling locations at the Potter Point ocean outfall and the reference site. Duplicate samples were collected where possible. 

2. The whole, unwashed samples of algae (Ulva sp.) and barnacle (Tesseropera rosea) were dried and ground prior to gamma spectrometry analysis. The fish, Luderick (Girella sp.), were cut into flesh fillets.

3. Fresh Weight (FW) Radioactivity is in units of becquerels per kilogram of fresh (wet) sample.

4. Be-7 and K-40 are of natural origin. 
5. < MDA indicates that the result was below the minimum detectable activity, calculated with 95% confidence. Indicative (median) MDA values for various radionuclides and environmental media are listed in Table 1.

TABLE 37. RADIOACTIVITY IN ALGAE, POTTER POINT AND THE ROYAL NATIONAL PARK, July 2003 to June 2004

	Location
	Date

Sampled 

 
	Gamma-emitters in ALGAE 

(Bq/kg FW) 

	
	
	Be-7
	K-40
	Am-241
	Co-60
	Cs-137
	I-131

	Potter Point
Ocean Outfall
	22-10-03
	< MDA
	191 ( 18
	< MDA
	< MDA
	< MDA
	2.6 ( 0.3

	
	15-6-04
	< MDA
	269 ± 26
	< MDA
	< MDA
	< MDA
	5.1 ± 0.5

	
	
	
	
	
	
	
	

	The Royal National Park
Reference
Site
	5-12-03
	6.1 ( 0.9
	140 ( 17
	< MDA
	< MDA
	< MDA
	< MDA

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	



