ANSTO E-745


TABLE 4

RADIOACTIVITY IN FISH FROM POTTER POINT 

OCEAN OUTFALL, 2000

	Sampling

Location (1)
	Date

Sampled
	Gamma-emitters in FISH (2)
Bq/kg FW (3)

	
	
	U&Th

 Series (4)
	40K
	60Co
	137Cs (5)
	131I

	POTTER

POINT

Ocean

Outfall
	
	
	
	
	
	

	
	12-5-00
	(
	130 ( 10
	< 0.7
	< 0.6
	0.81 ( 0.20

	
	
	
	
	
	
	

	
	5-12-00
	(
	140 ( 10
	< 0.5
	0.46 ( 0.13
	< 0.3

	
	5-12-00
	(
	150 ( 10
	< 0.5
	< 0.8
	< 0.3

	
	
	
	
	
	
	


Notes:

1. Samples were split to form duplicates, where possible. See Figure 4 for sampling locations.

2. Flesh fillets of Luderick (Girella sp.) were analysed.

3. Radioactivity is in units of becquerels per kilogram of fresh (wet) sample.

4. ( in the U&Th Series column indicates the unquantified presence of decay products from the natural uranium-238 or thorium-232 series. U&Th Series  and 40K are of natural origin.

5. Caesium-137 is widely dispersed in the environment from atmospheric weapons testing and is found at similar levels in other species of fish from Australian regional waters.

ENVIRONMENTAL and EFFLUENT MONITORING at ANSTO SITES, 2000

43

