ANSTO E-745


TABLE 7c

RADIOACTIVITY IN COMPOSITE 

WATER SAMPLES FROM MDP BUND C, 2000

Monthly Composite(1)
RADIOACTIVITY (Bq/L )


Gross ( (2)
Gross ( (3)
Gamma Emitters




7Be (4)
40K
137Cs (5)
Other

January
< 0.01(6)
0.58 ( 0.01
0.20 ( 0.04
0.17 ( 0.04
0.024 ( 0.004


February
0.03 ( 0.01
0.82 ( 0.01
0.28 ( 0.04
0.14 ( 0.03
0.059 ( 0.006
60Co  0.008 ( 0.003

March
< 0.03
0.51 ( 0.01
0.20 ( 0.03
0.18 ( 0.04
0.017 ( 0.004


April
< 0.03
0.38 ( 0.01
0.10 ( 0.03
0.13 ( 0.04
0.012 ( 0.003


May
< 0.02
0.32 ( 0.01
0.18 ( 0.03
0.10 ( 0.03
0.012 ( 0.002


June
< 0.02
0.28 ( 0.01
0.08 ( 0.03
0.16 ( 0.03
0.010 ( 0.003


July
< 0.02
0.28 ( 0.01
0.15 ( 0.03
< 0.093
< 0.021 


August
0.03 ( 0.01
0.46 ( 0.01
0.09 ( 0.03
0.11 ( 0.03
0.027 ( 0.004
241Am  0.010 ( 0.002

60Co  0.031 ( 0.006

September
0.03 ( 0.01
0.32 ( 0.01
0.21 ( 0.04
0.11 ( 0.05
0.012 ( 0.003


October
< 0.02
0.44 ( 0.01
0.25 ( 0.04
< 0.10
0.014 ( 0.003


November
0.03 ( 0.01
0.47 ( 0.01
0.18 ( 0.03
0.24 ( 0.03
0.013 ( 0.003


December
0.09 ( 0.01
1.21 ( 0.02
0.31 ( 0.05
0.16 ( 0.03
0.17 ( 0.02
60Co  0.043 ( 0.008

Notes:

1. Refer to Figure 2 for the sampling location. MDP Bund C was sampled weekly for tritium analysis (see Table 7b). The remainder of the weekly samples were combined to make a monthly composite water sample for gross alpha, beta and gamma analysis.

2. The NSW Clean Water Regulations (1972) specify limits for radioactivity in class C waters as follows:

Gross alpha (() 1.1 Bq/L ;  Gross beta (() 11.1 Bq/L.

3. The gross beta results include the contribution from potassium-40 (a natural beta-gamma emitter).

4. 7Be and 40K are of natural origin.

5. In 2000, the average concentration of 137Cs in MDP Bund Composites was < 0.020 Bq/L, which is < 0.2 % of the WHO reference value for 137Cs in drinking water (10 Bq/L). 

6. A less-than value indicates that the result was below the minimum detectable activity (stated at the 95 % confidence level).

ENVIRONMENTAL and EFFLUENT MONITORING at ANSTO SITES, 2000

47


